CIVE 202 – Civil Engineering Analysis II

Spring 2026

Civil and Environmental Engineering

Dr. Kaycie Lane

ANNOTATED CODE DOCUMENT

Project #3 – FHWA Data Visualization and IDM Simulation

To: Dr. Lane

From: Ayub Abdi

Date: March 30, 2026

Subject: Annotated Code Document for Project #3

This annotated code document explains the Python code used in the Project #3 notebook. Each annotation identifies the Python function being used and explains the purpose of that function in complete sentences. Variable names are carried through consistently so the workflow is easy to follow from data import, to cleaning, to plotting, to the Intelligent Driver Model simulation.

| Python Code | Annotations |
| --- | --- |
| import pandas as pd import numpy as np import matplotlib.pyplot as plt | You will see below how to import the pandas library, the numpy library, and the matplotlib.pyplot library into the Python programming environment. The alias of these libraries is as follows: pandas. pd numpy. np matplotlib.pyplot. plt Making use of these aliases will not only make it easier for you to chain commands quickly but will also make your code easier to read. |
| nhts_data = pd.read_csv('NHTS.csv') ngsim_data = pd.read_csv('NGSIM.csv') | The pd.read_csv() function is used by these lines to import the NHTS and NGSIM CSV files into a Python program as data frames. The variables nhts_data and ngsim_data hold the raw data sets so they can be viewed and analyzed. |
| print("NHTS preview:") print(nhts_data.head()) print("\nNGSIM preview:") print(ngsim_data.head()) | You can use the print() function to see labels and example output so that you can look at the data you are working with. You see the first five rows of each dataframe using the head() function, which allows you to ensure that the file loaded properly. |
| print("NHTS columns:") print(nhts_data.columns.tolist()) print("\nNGSIM columns:") print(ngsim_data.columns.tolist()) | The attribute of the columns gives back the columns’ labels for the dataframes, and applying the function tolist() to the columns returns the columns’ labels as a list; this way you can see all of the variables that are available before selecting which columns you want to use for the methods’ report. |
| nhts_subset = nhts_data[['travel_day', 'vehicle_age', 'vehicle_type']].dropna() ngsim_subset = ngsim_data[[ 'trajectory_number', 'Time', 'leader_speed(m/s)', 'follower_speed(m/s)', 'leader_acc(m/s^2)', 'follower_acc(m/s^2)', 'leader_position(m)', 'follower_position(m)' ]].dropna() | In this part, variable names from the methods report are selected using bracket selection and all other variable names are excluded from the data set. The dropna() function removes any observations that have missing values as well so that future plots or calculations only contain complete observations. |
| trajectory_number = 6 trajectory_data = ngsim_subset[ngsim_subset['trajectory_number'] == trajectory_number].copy() trajectory_data = trajectory_data.reset_index(drop=True) | A boolean condition applied to the trajectory_number will enable filtering the NGSIM data down to only one trajectory. The copy() function creates a new independent DataFrame containing that trajectory, and the reset_index() function rebuilds the row indices after filtering. |
| trajectory_data['spacing'] = ( trajectory_data['leader_position(m)'] - trajectory_data['follower_position(m)'] ) print(trajectory_data[['Time', 'leader_position(m)', 'follower_position(m)', 'spacing']].head()) | The new variable, spacing, is created by subtracting the follower’s position from the leader’s position at every time step. After calculating spacing, use head() to display the first rows so you can check on the new variable. |
| travel_day_counts = nhts_subset['travel_day'].value_counts().sort_index() plt.figure(figsize=(10, 6)) plt.bar(travel_day_counts.index, travel_day_counts.values, edgecolor='black') plt.xlabel('Travel Day') plt.ylabel('Number of Trips') plt.title('Number of Trips by Travel Day') plt.xticks(rotation=45) plt.tight_layout() plt.show() | The value_counts() function provides counts of the number of trips that were taken according to their travel day category, and the sort_index() function will sort those categories as listed through their three-letter label. The create bar chart functions utilize these counts in order to provide the bar chart, the chart has x and y axis labeled, the chart is titled, the rotation of tick marks has been adjusted for readability, and also there exists a tight layout for this chart to fit within the report specifications. |
| plt.figure(figsize=(10, 6)) plt.hist(nhts_subset['vehicle_age'], bins=20, edgecolor='black') plt.xlabel('Vehicle Age (years)') plt.ylabel('Frequency') plt.title('Distribution of Vehicle Age') plt.tight_layout() plt.show() | To create a histogram of the vehicle_age variable; we will be using the plt.hist() method. The `bins` argument will define how to group the numerical values into intervals, and the labeling functions will identify what the axes and the title of the histogram are so it will be easy to interpret the distribution. |
| plt.figure(figsize=(12, 6)) nhts_subset.boxplot(column='vehicle_age', by='vehicle_type', grid=False) plt.xlabel('Vehicle Type') plt.ylabel('Vehicle Age (years)') plt.title('Vehicle Age by Vehicle Type') plt.suptitle('') plt.xticks(rotation=45) plt.tight_layout() plt.show() | The vehicle_age grouped by vehicle_type are analysed using the boxplot() function enabling visual comparability of median, spread, and outlier value between grouping. The suptitle() function was used to remove the default group title for your plot. All other formatting functions were used strictly for aesthetic purposes to improve the overall quality of the resulting plot. |
| plt.figure(figsize=(12, 6)) plt.plot(trajectory_data['Time'], trajectory_data['leader_speed(m/s)'], label='Leader Speed', linewidth=2) plt.plot(trajectory_data['Time'], trajectory_data['follower_speed(m/s)'], label='Follower Speed', linewidth=2) plt.xlabel('Time (s)') plt.ylabel('Speed (m/s)') plt.title('Leader and Follower Speed vs Time') plt.legend() plt.grid(True, alpha=0.3) plt.tight_layout() plt.show() | In this example, we will be using the plt.plot() function again to plot the speed of the driver and follower on the same graph. The additions of the legend() and grid() functions aid in quickly comparing the two vehicles over time and to assist in supporting the time-series discussion in this report. |
| plt.figure(figsize=(12, 6)) plt.plot(trajectory_data['Time'], trajectory_data['spacing'], label='Spacing', linewidth=2) plt.xlabel('Time (s)') plt.ylabel('Spacing (m)') plt.title('Vehicle Spacing vs Time') plt.legend() plt.grid(True, alpha=0.3) plt.tight_layout() plt.show() | The plot that is created with plt is used to depict the spacing between both vehicles using the spacing variable; thereby demonstrating how the distance between them changes from the beginning of the observation period to the end (i.e., over time) on this chart (spatial distribution of vehicle spacing). |
| def idm_acceleration(v, s, delta_v, v0, s0, T, a, b, delta): s_star = s0 + v * T + (v * delta_v) / (2 * np.sqrt(a * b)) s_star = max(s_star, s0) acceleration = a * (1 - (v / v0) ** delta - (s_star / s) ** 2) return acceleration | This is a section where we will define a unique Python function with the name idm_acceleration() used for calculating Intelligent Driver Model (IDM) acceleration. The first operator inside the function, np.sqrt(), finds the square root value for use in IDM calculations; max() sets a lower limit on how far apart two vehicles should be from each other according to their desired distance, and finally, return will send an acceleration value to the simulation program. |
| time_data = trajectory_data['Time'].values leader_position = trajectory_data['leader_position(m)'].values leader_speed = trajectory_data['leader_speed(m/s)'].values real_follower_position = trajectory_data['follower_position(m)'].values real_follower_speed = trajectory_data['follower_speed(m/s)'].values real_follower_acc = trajectory_data['follower_acc(m/s^2)'].values dt = 0.1 v0 = 30 s0 = 2 T = 1.5 a = 1.0 b = 1.5 delta = 4 n_steps = len(time_data) sim_position = np.zeros(n_steps) sim_speed = np.zeros(n_steps) sim_acc = np.zeros(n_steps) sim_position[0] = real_follower_position[0] sim_speed[0] = real_follower_speed[0] | The attribute values transforms the columns you selected from your dataframe into arrays which are easier to utilize during the simulation loop. The len() function is used to determine the number of time steps; as a result the np.zeros() function produces a blank array for each of the three types of resulting values (position, speed, and acceleration) generated from the simulation loop. |
| for i in range(n_steps - 1): gap = max(leader_position[i] - sim_position[i], 0.1) delta_v = sim_speed[i] - leader_speed[i] sim_acc[i] = idm_acceleration(sim_speed[i], gap, delta_v, v0, s0, T, a, b, delta) sim_speed[i + 1] = max(sim_speed[i] + sim_acc[i] * dt, 0) sim_position[i + 1] = sim_position[i] + sim_speed[i] * dt print("Simulation complete.") | With the loop using the range() method to loop through the simulation one timestep at a time, the gap and updated speed will have no chance of being unrealistically small or negative because it is limited by the use of max() function. As each time step occurs, the idm_acceleration() function calculates the simulated follower acceleration.. |
| plt.figure(figsize=(12, 6)) plt.plot(time_data, leader_position, label='Leader Position', linewidth=2) plt.plot(time_data, real_follower_position, label='Real Follower Position', linewidth=2) plt.plot(time_data, sim_position, label='Simulated Follower Position', linewidth=2) plt.xlabel('Time (s)') plt.ylabel('Position (m)') plt.title('Real and Simulated Vehicle Position vs Time') plt.legend() plt.grid(True, alpha=0.3) plt.tight_layout() plt.show() | Measured vehicle locations are compared to simulated follower position from the IDM model. The three lines that have been plotted are meant to show the relationship between how the followers followed the leader versus how the simulators followed each other. |
| plt.figure(figsize=(12, 6)) plt.plot(time_data, real_follower_speed, label='Real Follower Speed', linewidth=2) plt.plot(time_data, sim_speed, label='Simulated Follower Speed', linewidth=2) plt.xlabel('Time (s)') plt.ylabel('Speed (m/s)') plt.title('Real and Simulated Follower Speed vs Time') plt.legend() plt.grid(True, alpha=0.3) plt.tight_layout() plt.show() | We can compare the speed of a real follower to that of a simulated follower's speed over time using the plt.plot() method in this graph. A comparison of speed over time between the speed of the follower and the simulated follower will validate how well the IDM is producing a realistic speed profile for real drivers compared with simulated drivers. |
| plt.figure(figsize=(12, 6)) plt.plot(time_data, real_follower_acc, label='Real Follower Acceleration', linewidth=2) plt.plot(time_data, sim_acc, label='Simulated Follower Acceleration', linewidth=2) plt.xlabel('Time (s)') plt.ylabel('Acceleration (m/s^2)') plt.title('Real and Simulated Follower Acceleration vs Time') plt.legend() plt.grid(True, alpha=0.3) plt.tight_layout() plt.show() | This section compares the observed follower acceleration to the acceleration generated by the simulation. The graph helps evaluate whether the IDM model captures changes in vehicle response over time. |
| real_gap = leader_position - real_follower_position sim_gap = leader_position - sim_position plt.figure(figsize=(12, 6)) plt.plot(time_data, real_gap, label='Real Gap', linewidth=2) plt.plot(time_data, sim_gap, label='Simulated Gap', linewidth=2) plt.xlabel('Time (s)') plt.ylabel('Gap Distance (m)') plt.title('Real and Simulated Gap Distance vs Time') plt.legend() plt.grid(True, alpha=0.3) plt.tight_layout() plt.show() | The first two lines subtract the follower position from the leader position to determine real_gap and sim_gap, respectively. The plotting functions are then used to compare real and simulated gap distances, which helps in evaluating the spacing behavior of the model. |